A Talker Uses 5 Commands

========================

$81 - read MCU registers 12 bytes (-$81,stackpointer,CC,B,A,IX,IY,PC)

$C1 - write MCU registers-send 9 bytes: [CC, B, A, IX, IY, PC] to MCU

$01 - read a memory block (1-256 bytes)

$41 - write a memory block (1-256 bytes)

$4A - allow a test program to continue from a breakpoint (SWI)

...all other "commands" just echo the 2s complement of the byte

"Inherent" Commands (high bit set)

==================================

$81 [1000 0001] - Transmit 9 MCU registers to PC (12 bytes with stack)

[7E inverted]  [7E, STKH-STKl,CC,B,A,IXH-IXL,IYH-IYL,PCH-PCL]

$C1 [1100 0001] - Overwrite 9 MCU registers with bytes read from PC

[3E inverted]    [CC,B,A,IXH-IXL,IYH-IYL,PCH-PCL]

These are the registers saved on the stack during an interrupt so

the MCU is forced to do a fake interrupt which pushes 9 bytes on

stack. The talker then reads the 9 bytes off the stack and transmits

Then to the PC. These should be a "frozen" picture of the MCU.

"Memory" Commands (high bit is clear)

=====================================

<command> <count> <addHigh> <addLow> <data bytes>

where <count>           0000 to 1111 (0=256 bytes)

<addHigh><addLow> 16 bit address

<data bytes>      bytes to write into memory [send <count> bytes]

$01 [0000 0001] [-FE] - Read <count> bytes

$41 [0100 0001] [-BE] - Overwrite <count> (hangs if you don't send enough)

Breakpoint Control of Programs Under Debug Testing

==================================================

Debugger (IDE) software on the PC inserts a one byte SWI instruction

into the memory location just before a "beakpoint". How the IDE deals

with added bytes in the machine code is often a trade secret. It may

dynamically re-assemble below the SWI, replace/restore bytes displaced by

the SWI, or just re-assemble and reload the entire program on every change.

When the SWI is encountered the 68HC11 transmits a $4A (BRKCODE) to the

host PC indicating a breakpoint has been reached then loops at SWIIDLE

waiting for a reply.

A byte from the PC causes an interrupt...escaping the SWIIDLE loop

and branching to SWISRV1.

If the PC sends a $4A (BRKACK) the 68HC11 falls through

...RTI returns to test program.

If the PC sends something other than a $4A (or a valid <command>) the 68HC11:

...transmits <addHigh><addLow> of the breakpoint address

...transmits a copy of the 9 MCU registers

...the PC byte is left in the SCI buffer for the test program to read

If the PC sends a valid <command> (but not a $4A) the 68HC11 will process

{$81, $C1, $01, $41} ...then RTI back to the test program